January 18, 2013

AEROLOG version 0.1

This document describe AEROLOG gauge for Microsoft Flight Simulator X and Microsoft Flight Simulator 2004: A Century of Flight

- The table describes all variables displayed and written by AEROLOG gauge

| No. | Variable | Description | Units | Comments |
| --- | --- | --- | --- | --- |
| 1 | Seconds | Time mark |  |  |
| 2 | Latitude | Geographic Latitude |  |  |
| 3 | Longitude | Geographic Longitude |  |  |
| 4 | Altitude | Pressured Altitude | Feet |  |
| 5 | Pitch | Plane pitch | Degrees | Positive when nose down |
| 6 | Bank | Plane bank | Degrees | Positive when rotate counterclockwise (on left wing) |
| 7 | Heading | Plane magnetic heading | Degrees |  |
| 8 | Vspeed | Vertical speed | Feet per minute | Positive when climb |
| 9 | RHeight | Radio height | Feet |  |
| 10 | Alpha | Plane body alpha | Degrees |  |
| 11 | Beta | Plane body beta | Degrees |  |
| 12 | IAS | Indicated airspeed | Knots |  |
| 13 | TAS | True airspeed | Knots |  |
| 14 | Mach | Mach number | Number |  |
| 15 | G-load | G load | In units of g-acceleration |  |
| 16 | Weight | Current weight | Pounds |  |
| 17 | CL | Total airplane lift coefficient |  |  |
| 18 | CD | Total airplane drag coefficient |  |  |
| 19 | BodyVX | Lateral (X-axes) ground velocity in aircraft BODY referenced frame | Feet per second |  |
| 20 | BodyVY | Vertical (Y-axes) ground velocity in aircraft BODY referenced frame | Feet per second | Positive in down direction |
| 21 | BodyVZ | Longitudinal (Z-axes) ground velocity in aircraft BODY referenced frame | Feet per second |  |
| 22 | BodyAX | Lateral (X-axes) ground acceleration in aircraft BODY referenced frame | Feet per second |  |
| 23 | BodyAY | Vertical (Y-axes) ground acceleration in aircraft BODY referenced frame | Feet per second | Positive in down direction |
| 24 | BodyAZ | Longitudinal (Z-axes) ground acceleration in aircraft BODY referenced frame | Feet per second | Positive when plane is accelerating |
| 25 | NEDVN | Velocity in North direction in inertial (NED) reference frame | Feet per second |  |
| 26 | NEDVE | Velocity in East direction in inertial (NED) reference frame | Feet per second |  |
| 27 | NEDVD | Velocity in Down direction in inertial (NED) reference frame | Feet per second |  |
| 28 | Q | Plane pitch rate | Degrees per second |  |
| 29 | P | Plane roll rate | Degrees per second |  |
| 30 | R | Plane yaw rate | Degrees per second |  |
| 31 | Qdot | Plane pitch acceleration | Degrees per second squared | Positive when nose down |
| 32 | Pdot | Plane roll acceleration | Degrees per second squared | Positive when rotate clockwise |
| 33 | Rdot | Plane yaw acceleration | Degrees per second squared | Positive when rotate clockwise |
| 34 | Elev | Elevator deflection | Degrees | Positive when the flight control deflected down |
| 35 | Ail | Aileron deflection | Degrees | Positive when left aileron deflected down (rotate clockwise) |
| 36 | Rudd | Rudder deflection | Degrees | Positive when the flight control deflected right |
| 37 | Trim | Elevator trim (stabilizer) deflection | Degrees | Positive when the flight control deflected down |
| 38 | Thrust | Engine #1 thrust | Pound-force |  |
| 39 | Lever | Engine #1 lever position | Percent | 0% - idle; 100% - full thrust |
| 40 | N1 | Engine #1 N1 | Percent |  |
| 41 | N2 | Engine #1 N2 | Percent |  |
| 42 | FF | Engine #1 Fuel flow | Pounds per second |  |
| 43 | ITT | Engine #1 Interstage Turbine Temperature | Degree Celsius |  |
| 44 | WindX | Wind at the aircraft in the lateral (X) axis | Meters per second |  |
| 45 | WindY | Wind at the aircraft in the vertical (Y) axis | Meters per second |  |
| 46 | WindZ | Wind at the aircraft in the longitudinal (Z) axis | Meters per second |  |
| 47 | WindSpd | Total ambient wind velocity | Meters per second |  |
| 48 | WindDir | Total ambient wind direction | Degrees, magnetic |  |
| 49 | Visibility | Current Visibility | Statue miles |  |
| 50 | theta | Standard temperature ratio (ambient air temperature divided by ISA air temperature) |  |  |
| 51 | delta | Standard pressure ratio (ambient air pressure divided by ISA air pressure) |  |  |
| 52 | sigma | Standard density ratio (ambient air density divided by ISA air density) |  |  |
| 53 |  |  |  |  |

- The axes frame used by the gauge and simulator

|  | Name of the Axis | Positive Direction | Type of rotary motion around the axis |
| --- | --- | --- | --- |
| X | Lateral | Starboard | Pitch |
| Y | Vertical | Upward | Yaw |
| Z | Longitudinal | Forward | Roll |